SECTION 08 42 29.33

SWINGING AUTOMATIC ENTRANCES

Specifier Note: This specification document shall serve as a guide specification for typical projects where the Stanley Access Technologies full energy swinging automatic entrance, with M-Force™ (Next Generation Magic-Force™) operators, and Swing-Guard® door mounted safety sensors, will be the basis of design. Specification must be reviewed for applicability on a per project basis. The specifier is directed to select appropriate options included herein. Consult with the local Access Technologies Territory Manager, when options, not specified, are required. See last page of this document for a summary of unspecified options.

- GENERAL

  - RELATED DOCUMENTS

    - Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

  - SUMMARY

    - This Section includes the following types of swinging automatic entrances:

      - Exterior and interior, swinging automatic entrances, full energy, with concealed header mounting.

      - Swinging automatic entrances shall include swing doors configured as follows:

        - Simultaneous pairs.

        - Single doors.

        - One-way traffic.

    - Related Sections:

      - Division 7 Sections for caulking to the extent not specified in this section.

      - Division 8 Section "Aluminum-Framed Entrances and Storefronts" for entrances furnished separately in Division 8 Section.

      - Division 8 Section "Door Hardware" for hardware to the extent not specified in this Section.

      - Division 8 Section Glazing for materials and installation requirements of glazing for automatic entrance doors.

      - Division 26 Sections for electrical connections provided separately including conduit and wiring for power to, and control of, swinging automatic entrances.

  - REFERENCES

    - General: Standards listed by reference, including revisions by issuing authority, form a part of this specification section to extent indicated. Standards listed are identified by issuing authority, authority abbreviation, designation number, title or other designation established by issuing authority.  Standards subsequently referenced herein are referred to by issuing authority abbreviation and standard designation.

    - Underwriters Laboratories (UL):

      - UL 325 – Standard for Door, Drapery, Gate, Louver, and Window Operators and Systems.

    - American National Standards Institute (ANSI)/Builders’ Hardware Manufacturers Association (BHMA):

      - ANSI/BHMA A156.10: Standard for Power Operated Pedestrian Doors.

    - American Society for Testing and Materials (ASTM):

      - ASTM B221 - Standard Specification for Aluminum and Aluminum-Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

      - ASTM B209 - Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

    - American Association of Automatic Door Manufacturers (AAADM):

    - National Fire Protection Association (NFPA):

      - NFPA 101 – Life Safety Code.

      - NFPA 70 – National Electric Code.

    - International Code Council (ICC):

      - IBC: International Building Code

    - Building Officials and Code Administrators International (BOCA), 1999:

    - International Standards Organization (ISO):

      - ISO 9001 - Standard for Manufacturing Quality Management Systems

    - National Association of Architectural Metal Manufacturers (NAAMM):

      - Metal Finishes Manual for Architectural and Metal Products.

Specifier Note: Modify paragraph below to suit project requirements. - Select appropriate standard finish from options below. - Make multiple selections as required; schedule accordingly. - See last page of this document for a summary of unspecified finish options. - Coordinate with other sections.

    - American Architectural Manufacturers Association (AAMA):

      - [AAMA 606.1 – Integral Color Anodic Finishes for Architectural Aluminum.]

      - [AAMA 607.1 - Clear Anodic Finishes for Architectural Aluminum.]

      - AAMA 611 Voluntary Specification for Anodized Architectural Aluminum.

      - AAMA 701 Voluntary Specification for Pile Weatherstripping and Replaceable Fenestration Weatherseals.

  - DEFINITIONS

    - Activation Device:  Device that, when actuated, sends an electrical signal to the door operator to open the door.

    - Safety Device:  Device that prevents a door from opening or closing, as appropriate.

  - PERFORMANCE REQUIREMENTS

    - General:  Provide swinging automatic entrances capable of withstanding loads and thermal movements based on testing manufacturer's standard units in assemblies similar to those indicated for this Project.

    - Operating Range:  Minus 30 deg F (Minus 34 deg C) to 130 deg F (54 deg C).

    - Opening-Force Requirements for Egress Doors: In the event power failure to the operator, swinging automatic entrance doors shall open with a manual force, not to exceed 30 lbf (133 N) applied at 1” (25 mm) form the latch edge of the door.

  - SUBMITTALS

    - General:  Submit listed submittals in accordance with Conditions of the Contract and Division 01 submittal procedures.

    - Shop Drawings:  Include plans, elevations, sections, details, hardware mounting heights, and attachments to other work. Indicate wiring for electrical supply.

    - Color Samples for selection of factory-applied color finishes.

    - Closeout Submittals: Provide the following with project close-out documents.

      - Owner’s Manual.

      - Warranties.

  - QUALITY ASSURANCE

    - Installer Qualifications:  Manufacturer's authorized representative, with certificate issued by AAADM, who is trained for installation and maintenance of units required for this Project.

    - Manufacturer Qualifications:  A qualified manufacturer with a manufacturing facility certified under ISO 9001.

    - Manufacturer shall have in place a national service dispatch center providing 24 hours a day, 7 days a week, emergency call back service.

    - Certifications: Swinging automatic entrances shall be certified by the manufacturer to meet performance design criteria in accordance with the following standards:

      - ANSI/BHMA A156.10.

      - NFPA 101.

      - UL 325 Listed.

      - IBC 2018.

      - BOCA.

    - Source Limitations:  Obtain swinging automatic entrances through one source from a single manufacturer.

    - Product Options:  Drawings indicate sizes, profiles, and dimensional requirements of swinging doors equipped with swinging automatic entrances and are based on the specific system indicated. Do not modify intended aesthetic effects, as judged solely by Architect, except with Architect's approval. If modifications are proposed, submit comprehensive explanatory data to Architect for review.

    - Power Operated Door Standard: ANSI/BHMA A156.10.

    - Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

    - Emergency-Exit Door Requirements:  Comply with requirements of authorities having jurisdiction for swinging automatic entrance doors serving as a required means of egress.

  - PROJECT CONDITIONS

    - Field Measurements:  General Contractor shall verify openings to receive swinging automatic entrances by field measurements before fabrication and indicate measurements on Shop Drawings.

    - Mounting Surfaces:  General Contractor shall verify all surfaces to be plumb, straight and secure; substrates to be of proper dimension and material.

    - Other trades:  General Contractor Advise of any inadequate conditions or equipment.

  - COORDINATION

    - Templates:  Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing swinging automatic entrances to comply with indicated requirements.

    - Electrical System Roughing-in: Coordinate layout and installation of swinging automatic entrances with connections to, power supplies, remote activation devices, and electric door latching hardware.

    - System Integration: Integrate swinging automatic entrances with other systems as required for a complete working installation. Where required for proper operation, provide a time delay relay to signal automatic door operator to activate only after electric lock system is released.

  - WARRANTY

    - Swinging automatic entrances shall be free of defects in material and workmanship for a period of one (1) year from the date of substantial completion.

    - During the warranty period the Owner shall engage a factory-trained technician to perform service and affect repairs. A safety inspection shall be performed after each adjustment or repair and a completed inspection form shall be submitted to the Owner.

    - During the warranty period all warranty work, including but not limited to emergency service, shall be performed during normal working hours.

- PRODUCTS

  - SWINGING AUTOMATIC ENTRANCES

    - Manufacturer: Stanley Access Technologies; swinging automatic entrance, with M-Force™ Series automatic door operator, and Swing-Guard™ door mounted safety system.

  - MATERIALS

    - Aluminum:  Alloy and temper recommended by manufacturer for type of use and finish indicated.

      - Headers, stiles, rails, and frames: 6063-T6.

      - Extruded Bars, Rods, Profiles, and Tubes:  ASTM B 221.

      - Sheet and Plate:  ASTM B 209.

    - Sealants and Joint Fillers:  Refer to Division 7 Section "Joint Sealants".

  - COMPONENTS

    - Framing Members:  Manufacturer's standard extruded aluminum reinforced as required to support imposed loads. Nominal size shall be 1 ¾ inch by 4 ½ inch (45 by 114 mm).

Specifier Note: Modify paragraph below to suit project requirements. - Select “Stile Design”; “Narrow” is standard. - Select “Bottom Rail Design”; 4 inch is standard. - Select “Muntin Bars”; “None” is standard.

    - Stile and Rail Door Panels:  Manufacturer's standard 1 ¾ inch (45 mm) thick glazed doors with extruded-aluminum tubular stile and rail members. Incorporate concealed tie-rods that span full length of top and bottom rails.

      - Glazing Stops and Gaskets:  Snap-on, extruded-security aluminum stops and preformed gaskets.

      - Stile Design: [Narrow stile; 2 inch (51 mm)] [Medium stile; 3 ½ inch (95 mm)] nominal width.

      - Bottom Rail Design:  Minimum [4 inch (102 mm)] [6 inch (152 mm)] [8 inch (203 mm)] [10 inch (254 mm)] [12 inch (305 mm)] nominal height.

      - Muntin Bars:  [None.] [Horizontal tubular rail member for each door; [2 inch (51 mm)] [4 1/4 inch (108 mm)] nominal height.]

      - Push/Pull Bars: Manufacturer’s standard.

    - Glazing:  Performed under Division 8 Section Glazing. All Glazing furnished by “by others” shall be 1/4 inch (6 mm) tempered, unless otherwise specified.

    - Header Case: Header case shall not exceed 6” (152 mm) square in section and shall be fabricated from extruded aluminum with structurally integrated end caps, designed to conceal door operators and controls. The operator shall be sealed against dust, dirt, and corrosion within the header case. Access to the operator and electronic control box shall be provided by a full-length removable cover, edge rabbetted to the header to ensure a flush fit.  Removable cover shall be secured to prevent unauthorized access.

    - Door Arms: A single straight arm, designed for top mounting in door panel, shall provide positive control of door through entire swing; units shall be specifically designed for use on center pivot doors.

    - Bottom Pivots: Units shall be specifically designed for use on center pivot doors of the size and weight required for the project and shall incorporate a surface applied tapered threshold plate.

Specifier Note: Modify paragraph below to suit project requirements. - Select appropriate thresholds for applications. - Make multiple selections as required; schedule accordingly.

    - Thresholds:  Manufacturer's standard thresholds as indicated below:

      - [Continuous standard tapered extrusion square by bevel, with bevel to exterior.]

      - [Continuous standard tapered extrusion square by bevel, with bevel to interior.]

      - [Continuous standard tapered extrusion double bevel.]

      - [Continuous standard square extrusion, for recessed installation.]

      - [No threshold.]

      - All thresholds to conform to details and requirements for code compliance.

    - Fasteners and Accessories:  Manufacturer's standard corrosion-resistant, non-staining, non-bleeding fasteners and accessories compatible with adjacent materials.

    - Signage:  Provide signage in accordance with ANSI/BHMA A156.10.

  - SWINGING DOOR OPERATORS

    - General:  Provide door operators of size recommended by manufacturer for door size, weight, and movement; for condition of exposure; and for long-term, maintenance-free operation under normal traffic load for type of occupancy indicated.

    - Electromechanical Operators:  Self-contained unit powered by a minimum 3/16 horsepower, permanent-magnet DC motor; through a high torque reduction gear system.

      - Operation:  Power opening and spring closing.

      - Operator Type: Full energy.

      - Handing: Non-handed; no tools required to change handing.

      - Capacity: Rated for door panels weighing up to 700 lb (318 kg).

      - Mounting: Visible

      - Features:

        - Adjustable opening and closing speeds.

        - Adjustable opening and closing force.

        - Adjustable back-check.

        - Adjustable hold-open time between 0 and 30 seconds.

        - Reverse on obstruction.

        - Time delay for electric lock integration.

        - Force compensation and closed loop speed control with active braking and acceleration.

        - Power Close.

        - Slam Protection.

        - Power Assist.

        - Lock Release.

        - Stall Sensor Ignore.

        - Electronic Coordination.

        - Optional Switch to open/Switch to close operation.

        - Optional push to activate operation.

        - Fire alarm interface, configurable to safely open or close doors on signal from fire alarm system.

    - Field Adjustable Spring Closing Operation: The operator shall close the door by spring energy employing the motor, as a dynamic brake to provide closing speed control. The closing spring shall be a helical compression spring, adjustable for positive closing action. The spring shall be adjustable, without removing the operator from the header, to accommodate a wide range of field conditions.

    - Independent Adjustable Closing and Latching Speed Control: The operator shall employ a rheostat module to allow for independent field adjustment of closing and latching speeds using the motor as a dynamic brake.

    - Field Adjustable Open Stop: The operator shall provide a field adjustable open stop to accommodate opening angles from 80 to 135 degrees without the need for additional components.

    - Consistent Cycle: The operator shall deliver an even, consistent open manual push force across the entire transition from door fully closed to door fully open. Additionally, the force shall be field adjustable to accommodate a wide range of on-site conditions.

    - Quiet Performance: The operator shall be designed to output audible noise ratios less than or equal to 50dba.

    - Manual Use: The operator shall function as a manual door closer in the direction of swing with or without electrical power. The operator shall deliver an even, consistent open force across the entire transition from door fully closed to door fully open.

    - Electrical service to door operators shall be provided under Division 26 Electrical.  Minimum service to be 120 VAC, 5 amps.

  - ELECTRICAL CONTROLS

    - Electrical Control System: Electrical control system shall include a microprocessor controller and a high-resolution position encoder. The encoder shall monitor revolutions of the operator shaft and send signals to microprocessor controller to define door position and speed.

      - The high-resolution encoder shall have a resolution of not less than 1024 counts per revolution. Systems utilizing external magnets and magnetic switches are not acceptable.

      - Electrical control system shall include a 24 VDC auxiliary output rated at 1 amp.

    - Performance Data: The microprocessor shall collect, and store performance data as follows:

      - Counter: A non-resettable counter to track operating cycles.

      - Event Reporting: Unit shall include non-volatile event and error recording including number of occurrences of events and errors, and cycle count of most recent events and errors.

      - LED Display: Display presenting the current operating state of the controller.

    - Controller Protection: The microprocessor controller shall incorporate the following features to ensure trouble free operation:

      - Automatic Reset Upon Power Up.

      - Main Fuse Protection.

      - Electronic Surge Protection.

      - Internal Power Supply Protection.

      - Resetable sensor supply fuse protection.

      - Motor Protection, over-current protection.

    - Power Close: When enabled, engages the operator to close a door that does not close completely at the end of a cycle.

    - Force Compensation: Utilizing the closed loop speed control, the operator shall maintain constant opening and closing speeds when subjected to excessive outside forces, such as positive or negative stack pressures.

    - Slam Protection: The operators speed control system prevents door from slamming at the full open or full closed position.

    - Power Assist: Operator mode that lowers opening forces when the door is used manually.  Power assist is active only while pushing or pulling the door.  The door will close when an opening force is no longer applied.

    - Lock Release: On doors with electric locking, operator shall include a closing function to release tension on a latch mechanism prior to opening the door.

    - Stall Sensor Ignore: Adjustable setting to disable swing side safety sensors at a specific angle.

    - Electronic Coordination: On pairs of doors, allows independent timing of opening and closing of each leaf as required for astragal coordination.

    - Soft Start/Stop: A “soft-start” “soft-stop” motor driving circuit shall be provided for smooth normal opening and recycling.

    - Obstruction Recycle: Provide system to recycle the swinging panels when an obstruction is encountered during the closing cycle.

    - Programmable Controller: Microprocessor controller shall be field programmable.

      - The following parameters may be adjusted:

        - Operating speeds and forces as required to meet specified ANSI/BHMA standard.

        - Adjustable and variable features specified.

      - Manual programming shall be available through local interface which has a two-digit display with a selection control including three push buttons.

    - Emergency Breakout Switch: A cam actuated emergency breakout switch shall be provided to disconnect power to the motor when an in-swinging door is manually pushed in the emergency out direction. The operator will then automatically reset, and power will be resumed.

    - Control Switch: Automatic door operators shall be equipped with a three-position function switch to control the operation of the door. Control switch shall provide three modes of operation, Automatic, Off, and Hold-Open.

    - Power Switch: Automatic door operators shall be equipped with a two position On/Off switch to control power to the door.

  - ACTIVATION AND SAFETY DEVICES

Specifier Note: Select one of two activation methods below. - Make multiple selections as required; schedule accordingly. - Coordinate with other sections.

    - [Push Plates: Provide 4 ½ inch (114 mm) square push plates with UL recognized SPDT switch.  Face plates and mounting studs shall be stainless steel.  Face plates shall be engraved with the international symbol for accessibility and “Push To Open”. Push plates shall be wall mounted in single or double gang electrical boxes and hardwired to door operator controls.]

    - [Motion Sensors:  Motion sensors shall be mounted on the approach side of door to detect pedestrians in the activating zone, and provide a signal to open doors. Units shall incorporate K-band microwave frequency to detect all motion in the activation zone.  Units shall be configurable for wide or narrow activation zones and shall be suitable for mounting heights up to 13 feet (4 m).  Motion sensors shall be equal to or better than Stanley Access Technologies SU-100.]

    - Presence Detection: Provide presence detection system designed to sense people in the swing zone when the swinging automatic entrance door is fully open, fully closed or in motion.  System provided shall consist of door mounted safety sensors and accessories required for a complete working system as follows:

      - Door Mounted Presence Detection Sensors: Door mounted presence detection sensors shall be reflective active infrared type designed specifically to sense moving or stationary objects in the swing zone on each side of a moving door leaf. Sensor housings shall be high impact shock resistant with tinted lenses suitable for door mounting.  Door mounted presence detection sensors shall not be affected by ultrasonic, ambient light or radio frequencies, within the vicinity of the swing door.

      - Secondary Activation: Presence detection system shall satisfy the requirements for secondary activation; no additional sensors shall be required.

      - Supporting relays and controllers shall be provided for a complete working system.

      - Door mounted presence detection sensors shall be equal to or better than Stanley Access Technologies Swing-Guard.

    - Presence Sensor Monitoring: Swinging automatic entrances control system shall include a means to verify the functionality of all active presence sensors in accordance with ANSI/BHMA A156.10. A detected fault shall cause automatic operation to cease until the fault is corrected.

Specifier Note: Retain “Guide Rails” section below as required for compliance with ANSI/BHMA A156.10. Refer to standard for requirements for alternative systems such as adjacent “walls”.

    - [Guide Rails: Guide rails shall be provided manufactured from 1/2 inch x 1 1/2 inch (12 mm x 38 mm) T-6063 alloy aluminum. Length and height shall be as required for compliance with ANSI A156.10. Finish shall match finish of automatic door operator headers.]

    - Finger Guards: Rubber or neoprene designed specifically for use with center pivot swinging automatic entrances, 1 inch (25 mm) in working section.

  - HARDWARE

    - General:  Provide units in sizes and types recommended by swinging automatic entrance and hardware manufacturers for entrances and uses indicated.

    - Deadlocks:  Manufacturer's standard deadbolt operated by exterior cylinder and interior thumb turn; with minimum 1 inch (25 mm) long throw bolt; ANSI/BHMA A156.5, Grade 1.

      - Cylinders:  As specified in Division 8 Section "Door Hardware."

      - Hook Latch: Laminated-steel hook, mortise type.

    - Weather Stripping:  Manufacturer's standard replaceable components complying with AAMA 701; made of wool, polypropylene, or nylon woven pile with nylon-fabric.

  - FABRICATION

    - General:  Factory fabricates swinging automatic entrance components to designs, sizes, and thickness indicated and to comply with indicated standards.

      - Form aluminum shapes before finishing.

      - Use concealed fasteners to greatest extent possible.

        - Where fasteners are subject to loosening or turning out from thermal and structural movements, wind loads, or vibration, use self-locking devices.

        - Reinforce members as required to receive fastener threads.

    - Door Panels:  Provide swinging automatic entrance door panels as prefabricated assemblies.

      - Fabricate tubular and channel frame assemblies with manufacturer's standard mechanical or welded joints.  Provide sub-frames and reinforcement as required for a complete system to support required loads.

      - Perform fabrication operations in manner that prevents damage to exposed finish surfaces.

      - Form profiles that are sharp, straight, and free of defects or deformations.

      - Prepare components to receive concealed fasteners and anchor and connection devices.

      - Fabricate components with accurately fitted joints with ends coped or mitered to produce hairline joints free of burrs and distortion.

    - Glazing:  Fabricate framing with minimum glazing edge clearances for thickness and type of glazing indicated.

    - Hardware:  Factory install hardware to the greatest extent possible; remove only as required for final finishing operation and for delivery to and installation at Project site.

  - ALUMINUM FINISHES

    - General: Comply with NAAMM Metal Finishes Manual for Architectural and Metal Products for recommendations for applying and designing finishes. Finish designations prefixed by AA comply with system established by Aluminum Association for designing finishes.

Specifier Note: Modify paragraph below to suit project requirements. - Select appropriate standard finish from options below. - Make multiple selections as required; schedule accordingly. - See last page of this document for a summary of unspecified finish options.

    - [Class II, Clear Anodic Finish: AA-M12C22A31 Mechanical Finish: as fabricated; Chemical Finish: etched, medium matte; Anodic Coating: Architectural Class II, clear coating 0.40 mils minimum complying with AAMA 611-98, and the following:

      - AAMA 607.1

      - Applicator must be fully compliant with all applicable environmental regulations and permits, including wastewater and heavy metal discharge.]

    - [Class I, Color Anodic Finish: AA-M12C22A42/A44 Mechanical Finish: as fabricated; Chemical Finish: etched, medium matte; Anodic Coating: Architectural Class I, integrally colored or electrolytically deposited color coating 0.70 mils minimum complying with AAMA 611-98, and the following:

      - Color: Dark Bronze.

      - AAMA 606.1

      - Applicator must be fully compliant with all applicable environmental regulations and permits, including wastewater and heavy metal discharge.]

- EXECUTION

  - INSPECTION

    - Examine conditions, with Installer present, for compliance with requirements for installation tolerances, header support, and other conditions affecting performance of swinging automatic entrance doors. Proceed with installation only after unsatisfactory conditions have been corrected.

  - INSTALLATION

    - General:  Do not install damaged components.  Fit joints to produce hairline joints free of burrs and distortion. Rigidly secure non-movement joints.

    - Mounting:  Install swinging automatic entrances/headers plumb and true in alignment with established lines and grades.  Anchor securely in place.

      - Install surface-mounted hardware using concealed fasteners to greatest extent possible.

      - Set headers, arms and linkages level and true to location with anchorage for permanent support.

    - Door Operators:  Connect door operators to electrical power distribution system as specified in Division 26 Sections.

    - Glazing:  Performed under Division 8 Section "Glazing" in accordance with swinging automatic entrance manufacturer’s instructions.

    - Sealants:  Comply with requirements specified in Division7 Section "Joint Sealants".

  - FIELD QUALITY CONTROL

    - Testing Services:  Factory Trained Installer shall test and inspect each swinging automatic entrance door to determine compliance of installed systems with applicable ANSI standards.

  - ADJUSTING

    - Adjust door operators, controls, and hardware for smooth and safe operation, for tight closure, and complying with requirements in ANSI/BHMA A156.10 by AAADM Certified Technician.

  - CLEANING AND PROTECTION

    - Clean surfaces promptly after installation.  Remove excess sealant compounds, dirt, and other substances.  Repair damaged finish to match original finish.

END OF SECTION 08 42 29.33

Available options not specified in this document are summarized as follows:

Traffic Pattern options (One-Way Specified)

Review and confirm additional safety requirements under ANSI/BHMA A156.10 for two-way traffic applications prior to specifying.

Integral transoms, with optional number and placement of verticals.

Finish options (Standard Options Specified):

Color anodizing options; “Champagne” to “Black”

Multi-coat Fluoropolymer painted finishes.

Cladding.

Locking options (Standard Options Specified):

3-Point Locks

Lock position indicators

Activation options (Standard Hard Wired Push Plates Specified) including:

Wireless push plates.

Wireless transmitters.

Push plate posts.

Contact your local Stanley Access Technologies representative for more information on specifying the right swinging automatic entrance for your project.

These specifications represent a “sample” configuration and depict design features that are commonly used. These specifications do not reflect “standard” features and are provided for informational purposes only. Please note that there is no standard “off the shelf” product.  Stanley custom manufactures each product to its customers’ specifications. It is the customer’s responsibility to validate that a particular configuration of Stanley’s products is suitable for a specific application.  All specifications and designs contained herein are subject to change without notice or obligation.